How to use your Raspberry Pi as a Wireless Access Point (WAP)

Start the Raspberry Pi, go to and to -> Preferences -> Raspberry Pi Configuration

Once the Raspberry Pi is on the first thing to do is to set up country, time zone, keyboard and WiFi to the appropriate location in this case United States eastern.

Figure 1. Starting Raspberry Pi configuration system

On the configuration windows, in Localisation:

- Set keyboard to English(US)

Set WiFi Country to US

Figure 2. Set WiFi Country Code                                   Figure 3. Set Timezone

Then follow steps 1 through 9 to configure the Raspberry Pi as WAP

Step 1: Update and Upgrade Raspbian: respectively with the following commands:

sudo apt-get update

sudo apt-get upgrade

Figure 4. Updating and upgrading the System

Once the upgrade is done reboot the system with following command:

sudo reboot

Step 2: Install hostapd and dnsmasq

hostapd and dnsmasq are the two programs we are going to use to turn the Raspberry Pi into WAP. hostapd allows us to use the built in WiFi as an access point. dnsmasq serves as both  Dynamic Host Configuration Protocol (DHCP) and Domain Name System (DNS) server

Note: Only type what is in red font do the same for all the steps

sudo apt-get install hostapd

sudo apt-get install dnsmasq

.

After the programs are installed turn them off before starting their configuration with the following commands:

sudo systemctl stop hostapd

sudo systemctl stop dnsmasq

Step 3: Configure a static IP for the wlan0 interface

For our purposes here, we’ll be using the standard home network IP addresses, like 192.168.###.###.

IP addresses are analogous to people’s assigned places in a room. They can be static (meaning permanent ) or dynamic (meaning temporary. every time a particular computer access the network the address can change)

So let’s assign the STATIC (permanent) IP address 192.168.0.10 to the wlan0 interface.

To do so we’ll edit ( open ) the dhcpcd configuration file with the following command.

sudo nano /etc/dhcpcd.conf

Once open,  add the following lines at the end of the file:

interface wlan0

static ip_address=192.168.0.10/24

denyinterfaces eth0

denyinterfaces wlan0

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key.

Step 4: Configure the DHCP server (dnsmasq)

Rename the default configuration file and write a new one:

sudo mv /etc/dnsmasq.conf /etc/dnsmasq.conf.orig

sudo nano /etc/dnsmasq.conf

And type the following 2 lines in there

interface=wlan0

dhcp-range=192.168.0.11,192.168.0.30,255.255.255.0,24h

And as always save and exit the file with Ctrl+X, followed by Y and Enter.

Step 5: Configure the access point host software (hostapd)

Open the hostapd config file with the command:

sudo nano /etc/hostapd/hostapd.conf

This should create a brand-new (empty) configuration file named “hostapd.conf ” in the “hostapd” folder in the “etc ” directory.

Then type in the following:

interface=wlan0

bridge=br0

hw_mode=g

channel=7

wmm_enabled=0

macaddr_acl=0

auth_algs=1

ignore_broadcast_ssid=0

wpa=2

wpa_key_mgmt=WPA-PSK

wpa_pairwise=TKIP

rsn_pairwise=CCMP

ssid=genNETWORK

wpa_passphrase=Stustudent

Note:  where I have “genNETWORK” and “Stustudent,” each student should come up with his own names. This is how you’ll join the Pi’s network from other devices. In addition, wpa_passphrase (password) should be at least 8 characters long

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key

Next we need to show the system the location of the configuration file:

enter the following command to edit hostapd

sudo nano /etc/default/hostapd

In this file, let’s track down the line that says #DAEMON_CONF=”” – delete the ” # ” and put the path to our config file in the quotes, so that the ligne looks as it shows in figure 5.

DAEMON_CONF="/etc/hostapd/hostapd.conf"

Figure 5. removing # and adding the path to hostpad.conf file

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key

Step 6: Set up traffic forwarding

The idea here is that when you connect to your Pi, it will forward the traffic over your Ethernet cable. So, we’re going to have wlan0 forward via Ethernet cable to your modem. Edit the sysctl config file with the following command:

sudo nano /etc/sysctl.conf

Now find this line:

#net.ipv4.ip_forward=1

… and delete the “#” – leaving the rest, so it just reads:

net.ipv4.ip_forward=1

Figure 6. Deleting #. This will uncomment the line

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key

Step 7: Add a new iptables rule

Next, we’re going to add IP masquerading for outbound traffic on eth0 using iptables. To do that let’s enter the following command:

sudo iptables -t nat -A POSTROUTING -o eth0 -j MASQUERADE

…and save the new iptables rule with the following command:

sudo sh -c "iptables-save > /etc/iptables.ipv4.nat"

To load the rule on boot, we need to edit the file “rc.local”  located in the “etc” directory with the following command:

sudo nano /etc/rc.local

… and then add the following line just above the line exit 0:

iptables-restore < /etc/iptables.ipv4.nat

Figure 7. Adding the above line to load the rules on boot

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key

Step 8: Enable internet connection

Now the Raspberry Pi should be acting as an access point to which other devices can connect. However, those devices can’t use the Pi to access the internet just yet. To make that possible, we need to build a bridge that will pass all traffic between the wlan0 and eth0 interfaces.

To build the bridge, let’s install a bridge utility with the following command:

sudo apt-get install bridge-utils

We’re ready to add a bridge (the br0 we declare in the hostapd.conf file in step 5):

sudo brctl addbr br0

Next, we’ll connect the eth0 interface to our bridge:

sudo brctl addif br0 eth0

Finally, let’s edit the interfaces file with the following command:

sudo nano /etc/network/interfaces

…and add the following lines at the end of the file:

auto br0

iface br0 inet manual

bridge_ports eth0 wlan0

Figure 8. Adding lines to the bottom of  interfaces file

Then we’ll save the file and exit the editor by pressing Ctrl+X, followed by Y and Enter key

Step 9: Reboot

Now that we’re ready, let’s reboot with sudo reboot.

Now the Pi should be working as a wireless access point. Try it out by hopping on another device and looking for the network name used back in step 5.